BASIC TECHNOLOGY JSS 3

THIRD TERM

## SCHEME OF WORK

Week 1: revision of last term’s work

Week2: fault detection

_ Definition (trouble shooting)

_ tools for fault detection

Week3: basic electronics

_ Basic electronic devices

_Types of electronics devices

Week4: basic electronics (continues)

_components

_uses

Week5: machine motion

_ linear motion

_linear arrangement to produce linear motion

_use of slides and slots in mechanical system

Week 6: machine motion

_rotary motion

_one way

_reversible rotary motion

7_12 revision & examination.

Week 1         revision

Linear and rotary motion components play a vital role in various machines and systems. These components facilitate precise movements and transformations in both linear and rotational directions. Here's a breakdown of some key components and their applications:

Linear Motion Components:

1. Air Cylinders: Pneumatic linear actuators driven by pressure differentials, commonly found in automation systems and manufacturing equipment.

2. Ball Screws: Devices converting rotary motion into linear motion or torque into thrust, essential in CNC machinery and robotics.

3. Ball Slides: Simple linear motion devices ensuring smooth and controlled movement, frequently used in precision instruments.

4. Ball Splines: Friction-free linear motion devices suitable for various applications due to their efficient coupling mechanisms.

5. Crossed Roller Slides: Linear motion devices consisting of rollers crossed at a 90° angle, providing stability and precision, commonly used in optical and measurement systems.

6. Dovetail Slides: Linear motion devices allowing precise motion along a straight path, essential in scientific instruments and industrial machinery.

7. Electric Linear Actuators: Actuators driven by motors and screws, enabling controlled linear movements, widely applied in automotive systems and healthcare devices.

8. Hydraulic Cylinders: Actuation devices utilizing pressurized hydraulic fluid for linear motion, commonly found in heavy machinery and construction equipment.

9. Lead Screws and ACME Screws: Components driving linear motion through direct contact between the screw and the nut, essential in woodworking machinery and 3D printers.

10. Linear Bearings: Bearings enabling straight-line movement, frequently used in conveyor systems and industrial automation.

11. Linear Stages: Devices with a drive mechanism ensuring controlled and precise positioning, extensively used in semiconductor manufacturing and research laboratories.

12. Multi-Axis Positioning Systems: Complex systems employing linear, rotary, and goniometric stages to create customized positioning solutions for research and industrial applications.

13. Piezoelectric Actuators: Devices generating small but powerful displacements, commonly used in nanotechnology and micro-manipulation applications.

Rotary Motion Components:

1. Electric Rotary Actuators: Actuators powered by electromagnetic force, enabling rotational movement and precise control, found in robotics and automated assembly lines.

2. Hydraulic Rotary Actuators: Actuators utilizing pressurized hydraulic fluid for rotational motion, commonly applied in heavy machinery and industrial processes.

3. Pneumatic Rotary Actuators: Actuators driven by pressurized air, providing rotational movement, widely used in automation and valve control systems.

4. Rack and Pinion Drives: Drives converting rotational motion into linear movement via a rack and pinion combination, essential in machine tool positioning and robotics.

5. Roller Screws: Devices converting rotary motion into precise linear motion, suitable for applications requiring high accuracy and reliability, such as aerospace systems and medical devices.

Examples of Machines Utilizing Linear Motion:

1. CNC Machines: Computer Numerical Control machines rely on precise linear movements for cutting, shaping, and engraving various materials.

2. Automated Conveyor Systems: Linear motion components are integral in conveyor belts for transporting goods in manufacturing and distribution centers.

Machines Converting Linear to Rotary Motion:

Electric Motors: Electric motors often transform linear motion into rotational motion, which is then utilized in numerous appliances and industrial machinery.

2. Generators: Generators, particularly in power plants, convert linear motion, often from steam or water, into rotary motion to generate electricity.

Week 2: Fault Detection

Fault detection, isolation, and recovery (FDIR) constitute a pivotal domain within control engineering, focusing on systematically monitoring systems. This involves identifying faults, pinpointing their type, and determining their location. Two distinct approaches are employed: direct pattern recognition of sensor readings indicating faults and analyzing discrepancies between sensor readings and expected values derived from models. In the latter, a fault is recognized when the discrepancy or residual surpasses a specific threshold. Subsequently, fault isolation categorizes the fault type and its machinery location. Fault detection and isolation (FDI) techniques can be broadly classified into two categories: model-based FDI and signal processing-based FDI.

Model-Based FDI:

Model-based FDI relies on mathematical models and logic. For instance, in aircraft elevators, mathematical models help identify faults and transient issues.

Signal Processing-Based FDI:

In signal processing-based FDI, mathematical or statistical operations are performed on measurements. Techniques like time domain reflectometry use signal analysis to detect faults, where reflected signals are compared to the original to identify discrepancies. Clustering methods are employed to identify novel faults and segment signals into normal and faulty segments.

Machine Fault Diagnosis:

Machine fault diagnosis, a subset of mechanical engineering, focuses on identifying faults in machines, especially rotating machinery. Various data collection methods, including vibration monitoring and thermal imaging, are utilized. Data are then processed using techniques like spectral analysis, wavelet transform, and correlation methods. Root cause failure analysis is crucial; identifying the initial cause of the fault ensures precision maintenance. Fault diagnosis approaches can be broadly categorized as linear methods and quadratic methods. While linear methods are suitable for signal processing due to their invertibility, quadratic methods describe energy distribution in the time-frequency domain, aiding analysis and feature detection.

Robust Fault Diagnosis:

Real-world challenges, such as model uncertainties and measurement noise, complicate fault detection. Addressing industrial needs cost-effectively involves strategic implementation of fault diagnostics, considering maintenance strategies like condition-based maintenance,

planned preventive maintenance, preventive maintenance, and integrated vehicle health management.

Questions:

1. Explain the methods used for fault detection in machines.

2. Identify two techniques for fault detection.

Week 3   basic Electronics

What is Electron Emission?

In the realm of basic electronics, the phenomenon of electron emission holds significant importance. Within metals, electrons possess the mobility to shift between atoms, thereby contributing to the material's exceptional electrical conductivity. However, these electrons are confined within the metal's confines due to the attractive forces in play. When these negatively charged particles, electrons, attempt to escape the metal, the metal surface gains a positive charge. This electrostatic attraction between positive and negative charges pulls the electrons back into the metal, creating an equilibrium. The metal's surface acts as a barrier, preventing the escape of these free electrons.

Nevertheless, this surface barrier can be overcome by imparting a specific minimum amount of energy to the free electrons. This additional energy elevates their kinetic energy, enabling them to break free from the metal surface. Termed the work function of the metal, this minimum energy threshold is crucial. When this required energy, the work function, is supplied to the metal, electrons are liberated from the metal surface, a phenomenon known as electron emission.

The work function of a metal is contingent upon various factors, including the metal's inherent properties, its purity, and the nature of its surface. Electron emission becomes feasible when an external energy source, equivalent to the metal's work function, is supplied. This energy can manifest in the form of heat energy, light energy, or other forms. Depending on the energy source, electron emission manifests in several types:

1. Thermionic Emission: In this type, the metal is heated to a specific temperature, enabling free electrons to escape from its surface.

2. Field Emission: This type involves applying a robust electric field to the metal, which, due to the positive field's attraction, pulls electrons out from the surface.

Photoelectric Emission

Stay tuned with byju’s to learn more about electron emission and other concepts in quantum mechanics with interesting video lectures

QUESTIONS

1.List 4 types of methods of liberating electrons

2. explain thermionic emission

Top of Form

Week 4

Components of electronics

Various electronic components.

An electronic component is any basic discrete device or physical entity in an electronic system used to affect electrons or their associated fields. Electronic components are mostly industrial products, available in a singular form and are not to be confused with electrical elements, which are conceptual abstractions representing idealized electronic components.

Electronic components have a number of electrical terminals or leads. These leads connect to create an electronic circuit with a particular function (for example an amplifier, radio receiver, or oscillator). Basic electronic components may be packaged discretely, as arrays or networks of like components, or integrated inside of packages such as semiconductor integrated circuits, hybrid integrated circuits, or thick film devices. The following list of electronic components focuses on the discrete version of these components, treating such packages as compo

Various electronic components.

An electronic component is any basic discrete device or physical entity in an electronic system used to affect electrons or their associated fields. Electronic components are mostly industrial products, available in a singular form and are not to be confused with electrical elements, which are conceptual abstractions representing idealized electronic components.

Electronic components have a number of electrical terminals or leads. These leads connect to create an electronic circuit with a particular function (for example an amplifier, radio receiver, or oscillator). Basic electronic components may be packaged discretely, as arrays or networks of like components, or integrated inside of packages such as semiconductor integrated circuits, hybrid integrated circuits, or thick film devices. The following list of electronic components focuses on the discrete version of these components, treating such packages as components in their owner right

Question

1.list 5 componects of electronis

2.what is the function of amplier

Week 5  and  6                machine motion

# Rotary to Linear Motion

A slider-crank mechanism is a typical design which converts rotary motion into linear motion. It is achieved by connecting a slider and a crank with a rod. This mechanism is also utilized as a system that converts the reciprocating linear motion of an automobile engine into rotary motion.

The second figure (Fig b) is an example of the mechanism that has the same functions as the slider-crank in the first figure (Fig a) in addition to the sliding stroke adjustment feature for the slider. To add this feature, the sliding stroke adjusting screw is placed on top of the rotation shaft center of the rotation disk. The sliding stroke can be adjusted by the adjusting nut located on one end of the sliding stroke adjusting screw.

In addition, if high-speed rotation or operation for long hours is required, it is necessary to consider design items related to the reliability matters described here.

|  | 1. | Rotation balance of rotating body (the entire structures on the disk) |
| --- | --- | --- |
|  | 2. | Strength of rotating shaft |
|  | 3. | Prevention of adjusting nut-loosening (e.g. double-nut system) |
|  | 4. | Selection of wear-resistance parts for the area subject to wear |

The figure below illustrates the mechanism with exchangeable rod/slider parts. A U-shaped hook is installed so that the tip of the rod can be easily connected to the hinge pin at the hinged end of the crank.

#### Application Examples

For simple automation devices or fixtures made compatible with multiple models by connecting a processing unit to the slider unit, exchanging the slider unit after preparing a processing unit off-line can minimize the time required for mode switching.

|  | 1. | The drive mechanism of the squeegee unit for a simple screen printing machine: When materials are replaced owing to the pot life or when printing materials are replaced, it is possible to switch models quickly if you remove the slider parts with the squeegee integrated and exchange it with the slider parts with its squeegee position adjusted off-line. |
| --- | --- | --- |
|  | 2. | Simple press mechanism with multi-model compatibility |

#### Application Challenges

Because of this simple coupling method, where only a U-shaped hook is placed over the rotating crank, this mechanism is not compatible with the following types of motion:

|  | 1. | A high-speed rotating crank. The self-weight of the rod may not be sufficient to follow the motion. |
| --- | --- | --- |
|  | 2. | A large rotating radius. Motion from the crankshaft to the U-shaped hook will not be transmitted effectively in some areas. |
|  | 3. | Motion involving unstable speed or oscillation. The U-shaped hook may fall off. |

#### How to use a slider-crank mechanism

Below is an automation clever mechanism that converts crank rotation into linear motion and makes the linear motion stroke twice that of the original one.

In this structure, the slider used in the standard slider crank mechanism a toothed gear. In addition, the slider guide is separated into a fixed rack and a movable rack. The crank motion on the driving shaft is transmitted to the toothed gear. Then, the stroke of rolling motion (L) over the fixed rack increases by 100% (2L) when it reaches the gear top. This stroke (2L) acts on the movable rack installed on the gear top.

#### More Application Examples

The structure tends to be longer than originally intended if a pneumatic cylinder and a linear guide are used or if a structure where ball screws are used for connecting a motor is adopted. This automation clever mechanism is an effective solution for making the fixture short and compact.

- Oscillating motion of a processing machine

- Reciprocating linear motion mechanism for one-piece flow production fixture
(printing, wiping, and pressuring)

- Inspection jigs

Rotary motion can be converted into linear motion by using a screw. This volume introduces an automation clever mechanism that allows control of linear motion in various ways by adopting different types of screw structures. “Threaded” and “tapped” screws are used as a cam.

If the handle installed on the right edge is rotated, it causes a linear motion of the slide block placed on the two tapped screws that are assembled to the opposing two screws. This structure can also be applied for a double-speed mechanism, where single rotation of the handle is transformed into a movement of twice the pitch of the rotation.

#### Application Examples

|  | 1. | Electric terminal positioning or scanning movement for inspection equipment |
| --- | --- | --- |
|  | 2. | Adjustment mechanism for edge positioning of jigs compatible with various products |
|  | 3. | Double-speed mechanism |

#### Cams

A cam is the typical mechanical component used in the rotary-linear motion conversion mechanism. In this volume, we will look into application examples of the cam.

Cams are the excellent choice because of the following characteristics:

|  | 1. | Motion characteristics as speed, acceleration, and forces can be freely controlled by the output end to which the cam motion is transmitted. |
| --- | --- | --- |
|  | 2. | When combined with the link mechanism, incorporating a cam into the structure allows you to design a compact, lightweight yet highly rigid mechanism in a simple structure. |
|  | 3. | Cams can shorten the overall cycle time as they can overlap and control multiple movements. |
|  | 4. | Highly reliable. |

Based on these characteristics, cams are adopted in various applications including the terminal press-fitting mechanism of a high-speed terminal press-fitting machine, as well as the high-speed and complex timing control of an air release valve for an automobile engine. (See [Photo 1].) In the photo below, displacement of the plate cam is enlarged by the link mechanism. To ensure the high-speed responses, springs to prevent jumping have been installed.

A cam introduced here as the basics of automation clever mechanisms works as a supportive element that allows for quick attachment/detachment of fixtures by transforming the linear motion displacement into forces, rather than converting rotary motion into linear motion, which was explained earlier as a superior characteristic of cams used in motion control.
In , the rotary-linear motion conversion mechanism using a typical plate cam and a reciprocating follower is shown left and the one-touch clamp mechanism next to it.

Rotary to linear motion conversion can be configured with a variety of components and achieved with varying degrees of accuracy and strength. We hope that you have been inspired by these simple mechanisms.

### Rotary to Linear Motion

QUESTIONS

1.Define motion in engineering

2.Differentiate between linear motion and rotary motion

Top of Form

*

e

Bottom of Form

Top of Form

Bottom of Form

Top of Form

Bottom of Form